How to reset a BCM system via a wired or IP Phone

We had a problem in that a BCM system wasn’t accessible via the Element Manager program. A direct or dial up link made no difference, even via the 2 LAN IP address’s etc!. The odd thing was that access via the browser and monitor was OK. Also a dial up data connection was stable, until you tried using the Element Manager program and the screen kept showing “resolving cartridge”

I had a moment of madness and wondered if I could do anything via the internet phone. I managed to make a dial up data call and then I altered my I2050 soft phone version 2 to create a new profile for the Customers site as they all had just one IP licence available.

I was able to get one phone to auto assign, using the default passcode of 2264 (BCMI).  Because I was on a 28k data call it wasn’t enough to allow any dial tone, so don’t bother trying the intercom key!.

Anyway, key in the “Feature” key followed by 9*8. The User ID is the default Nortel telset one of ID = 738662 (setnna) and password = 266344 (config). Just click on the “NEXT” button 3 times to see “System Restart” appear. Click OK and Yes to reboot the BCM.

It’s as easy as that!. Wait 10 minutes before you try to log in again.