FEATURE CODES USED ON MERIDIAN NORSTAR

NOTE!

"FEATURE" BUTTON ON TERMINAL = "F"

UNLISTED FEATURE = "!"

"(XX)" NEXT TO F**    = THE  "COUNT FEAT"  NUMBER WHICH IS FOUND IN THE MISCELLANEOUS SECTION OF DEBUG (00-99).

(57)F** 7827887(STARTUP) = STARTUP OF DEFAULT PROGRAMMING

(58)F** 266344 (CONFIG)  = CONFIGURATION PROGRAMMING

(59)F** 23646 (ADMIN)    = ADMINISTRATION PROGRAMMING

(62)F** 33284 (DEBUG)    = DEBUGGING SECTION

F** 5463 (LINE)        = VIEW LINE SETTINGS (MODULAR ONLY)

F** 3986 (EXTN)      = VIEW EXTN SETTINGS (MODULAR ONLY)

(36)F* 0                   = KEY INQUIRY (ON ALL MEMORY KEYS)

(37)F* 1                   = EXTERNAL AUTODIAL ONTO MEMORY KEY

(38)F* 2                   = INTERNAL AUTODIAL ONTO MEMORY KEY

(39)F* 3                   = STORES FEATURES ONTO MEMORY KEYS

(40)F* 4                   = STORES PERSONAL SPEED DIAL NoS

!(00)F* 500               =       N/A

!(41)F* 501               = ENGLISH LANGUAGE IS PUT ON PHONE

!(42)F* 502               = ENGLISH LANGUAGE IS PUT ON PHONE

!     (F* 502)"MODULAR 30A" = "IN OPERATING" APPEARS ON DISPLAY

!(43)F* 503               = ENGLISH LANGUAGE IS PUT ON PHONE

!     (F* 503)"MODULAR 30B" = "IN OPERATING" APPEARS ON DISPLAY

!(00)F* 504               =       N/A

!(00)F* 505               =       N/A

!(00)F* 506               =       N/A

!(00)F* 507               =       N/A

!(00)F* 508               =       N/A

!(00)F* 509               =       N/A

(44)F* 6                   = RING TYPE (4 TYPES OF PHONE RING)

(45)F* 7                  = CONTRAST ADJUSTMENT ON TERMINALS

(46)F* 80                = RING VOLUME LEVEL ON TERMINALS

F* 81                = MOVING LINE KEYS ON TERMINALS

F* 82                = DIALLING MODES 3 TYPES TO CHOOSE

!(07)F* 83                =       N/A

!(07)F* 84                =       N/A

!(07)F* 85                =       N/A

!(07)F* 86                =       N/A

!(07)F* 87                =       N/A

!(07)F* 88                =       N/A

F* 89                = PROGRAMMED RELEASE (SAME AS RLS)

(47)F* 9                 = RUN/STOP (SETS UP A BREAK POINT)

(00)F 0                   = SPEED DIAL NUMBERS (01 TO 94)

(01)F 1                   = MESSAGE SEND TO ANOTHER TERMINAL

(48)F HASH 1       = CANCEL MESSAGE SEND

(02)F 2                   = RING AGAIN TO A PHONE WHEN FREE

(49)F HASH 2       = CANCEL RING AGAIN

(03)F 3                   = CONFERENCE CALL SETUP

(50)F HASH 3       = CANCEL CONFERENCE

(04)F 4                   = CALL FORWARD TO ANOTHER EXTENSION

(51)F HASH 4       = CANCEL CALL FORWARD

(05)F 5                   = LAST NUMBER REDIAL

(06)F 60                 = PAGE GENERAL

(07)F 61                 = PAGE INTERNAL USING TELE SPEAKERS

(08)F 62                 = PAGE EXTERNAL USING EXTR SPEAKER

(09)F 63                 = PAGE EXTERNAL AND INTERNAL

(10)F 64                 = LINE POOL GROUP LINES UNDER KEY

(11)F 65                 = MESSAGE REPLY CALL BACK TO PHONE

(52)F HASH 65     = CANCEL MESSAGE REPLY

(12)F 66                 = VOICE CALL WITHOUT PHONE RINGING

(13)F 67                 = SAVED NUMBER REDIAL WHILE ON CALL

(14)F 68                 = CLASS OF SERVICE PASSWORDS

(15)F 69                 = PRIORTY CALL

(16)F 70                 = TRANSFER CALL TO ANOTHER PHONE

(17)F 71                 = RECALL (USED ON PBX OR HOST EXCH)

!(07)F 72                 =            N/A

!(19)F 73                 =            N/A

(20)F 74                 = CALL PARK CALL RETRIEVED ANY EXTN

(21)F 75                 = CALL GROUP PICKUP IN PICKUP GRPS

(22)F 76                 = CALL PICKUP DIRECTED TO A EXTN

(23)F 77                 = CALL DURATION TIMER OF LAST CALL

(24)F 78                 = PAUSE (INSERTS A 3.5 SEC DELAY)

(25)F 79                 = HOLD EXCLUSIVE HOLD AT TERMINAL

(80)F 800                = TRUNK ANSWER ON CALLS IN N/S

(80)F 801                = CALL QUEUING (ANSWERS NEXT CALL)

(82)F 802                = GROUP LISTENING (SPEAKER&HANDSET)

F HASH 802    = CANCEL GROUP LISTENING

(83)F 803                = SHOW TIME & DATE WHILE ON A CALL

(80)F 804                = WAIT FOR DIAL TONE BEFORE DIAL'NG

!(85)F 805                =            N/A

!(86)F 806                =            N/A

!(87)F 807 "V2.9 COMPACT" = RINGING CALL (FUNCTION NOT KNOWN)

(88)F 808                = LONG TONES GIVEN WHEN KEY PRESSED

!(89)F 809                =            N/A

!(90)F 810                =            N/A

!(91)F 811                =            N/A

!(92)F 812                =            N/A

!(93)F 813                =            N/A

!(94)F 814                =            N/A

!(95)F 815                =            N/A

!(96)F 816                =            N/A

!(97)F 817                =            N/A

!(98)F 818                =            N/A

!(99)F 819                =            N/A

(08)F 82                  = CAMP ON (RE-ROUTE A CALL IF BUSY)

(29)F 83                  = PRIVACY ALLOWS EXTN SHARING ON LN

F HASH 83       = CANCEL PRIVACY

(30)F 84                  = LINE REDIRECTION (OVER AC15 PWS)

F HASH 84       = CANCEL LINE REDIRECTION

(31)F 85                 = DO NOT DISTURB

(53)F HASH 85      = CANCEL DO NOT DISTURB

(32)F 86                 = BACKGROUND MUSIC (IF FITTED)

(54)F HASH 86      = CANCEL BACKGROUND MUSIC

(33)F 87                 = SERVICE MODES (3 TYPES OFF & ON)

(55)F HASH 87     = CANCEL SERVICE MODES

F 871     "ON E-NET" = ALTERNATIVE RINGING ON SVCE MODES

F HASH 871"ON E-NET" = CANCEL ALTERNATIVE RINGING

F 872     "ON E-NET" = ALTERNATIVE RESTRICTIONS 3 SETS.

F HASH 872"ON E-NET" = CANCEL ALTERNATIVE RESTRICTIONS.

(34)F 88                 = VOICE CALL DENY(PHONE RINGS ONLY)

(56)F HASH 88     = CANCEL VOICE CALL DENY

!(08)F 89                 =            N/A

(67)F 900 TO F 999       = ACCOUNT FEATURE CODE  (FOR CLI)

(67)F 9**(RADMIN)        = RAD (REMOTE ACCESS DEVICE) FITTED

(67)F 9*0                = TURNS ON MODEM FOR RAD IF FITTED

(67)F 9*1                = STARTALK (VOICEMAIL FITTED)

(67)F 9*2                = CLI (CALL LOGGER INTERFACE)FITTED

NOTE!

PROGRAMMING  THE REMOTE  ACCESS DEVICE  CAN ONLY BE DONE FROM A M7310 OR M7324 TERMINAL, BUT THE MODEM  CAN BE  TURNED ON OR OFF FROM ANY PHONE WHEN CONNECTED TO A EXCHANGE LINE OR PRIVATE WIRE (AC 15).

ALSO  WHEN  USING ANY  OF THE  "FEATURE 9*?"  CODES, THE DISPLAY  WILL  SHOW  "INACTIVE FEATURE"  IF YOU SELECT A CODE WHICH DOES NOT HAVE A DEVICE CONNECTED. EG 9*2 = CALL LOGGER.

THE CALL LOGGER CANNOT BE PROGRAMMED WITH A M7100 PHONE.

DOCUMENTATION WRITTEN BY D K BROMLEY.        (VERSION 121094)